# Test Execution Logging Design

SoC Validation and Test Automation

2026-09-18

Table of contents

# 1. Test Execution Logging Design

## 1.1 1. Purpose

This document defines the architecture, format, collection, storage, processing, analysis, and lifecycle of test execution logs generated during SoC validation.

The objective is to provide:

- complete traceability of every test execution;

- a consistent logging mechanism across different test types;

- correlation between test actions and SoC responses;

- preservation of raw diagnostic information;

- machine-readable data for automated analysis;

- human-readable logs for debugging;

- integration with CI/CD and test reports;

- reproducibility of failures;

- long-term historical analysis and regression detection.

The logging system should support both:

- interactive/local execution, and

- automated CI execution.

# 2. 2. Scope

The logging architecture should cover the complete execution chain:

Test Specification
       |
       v
   Test Runner
    (pytest)
       |
       v
 Test Framework
       |
 +-----+-----------+
 |     |           |
 v     v           v
UART  JTAG/SWD   Board Control
 |     |           |
 +-----+-----------+
       |
       v
      SoC
       |
       v
 Measurements / Responses
       |
       v
 Log Collector
       |
 +-----+-------------+
 |     |             |
 v     v             v
Raw   Structured   Artifacts
logs    events
       |
       v
 Analysis / Reporting
       |
 +-----+-------------+
 |     |             |
 v     v             v
JUnit  HTML/PDF    Database

# 3. 3. Design Goals

These should be treated as explicit requirements.

## 3.1 3.1 Functional Requirements

| ID | Requirement |
| --- | --- |
| LOG-001 | Every test execution shall have a unique Execution ID |
| LOG-002 | Every test case shall have a unique Test ID |
| LOG-003 | All log entries shall be associated with an Execution ID |
| LOG-004 | Logs from different interfaces shall be correlatable |
| LOG-005 | Raw interface output shall be preserved |
| LOG-006 | Log entries shall contain timestamps |
| LOG-007 | Test actions shall be logged |
| LOG-008 | Test verdicts shall be logged |
| LOG-009 | Failures shall contain sufficient diagnostic information |
| LOG-010 | Logs shall be machine-readable |
| LOG-011 | Logs shall be human-readable |
| LOG-012 | Logs shall be usable outside the CI environment |
| LOG-013 | Test artifacts shall be associated with the execution |
| LOG-014 | Logging shall have minimal impact on test execution |
| LOG-015 | Logs shall support long-term historical analysis |

# 4. 4. Logging Architecture

The logging system should be separated into four logical layers.

+------------------------------------------------+
|              Test Execution Layer              |
|                                                |
| pytest / test cases / fixtures / assertions   |
+------------------------+-----------------------+
                         |
                         v
+------------------------------------------------+
|               Automation Layer                 |
|                                                |
| Test actions / drivers / framework events     |
+------------------------+-----------------------+
                         |
          +--------------+--------------+
          |              |              |
          v              v              v
       UART           JTAG/SWD       Board HW
       Logs           Logs           Logs
          |              |              |
          +--------------+--------------+
                         |
                         v
+------------------------------------------------+
|             Log Collection Layer               |
|                                                |
| Raw logs + Structured events + Metadata       |
+------------------------+-----------------------+
                         |
                         v
+------------------------------------------------+
|          Storage / Analysis Layer              |
|                                                |
| Files / DB / CI artifacts / Reports            |
+------------------------------------------------+

This separation is important because raw logs and interpreted logs should not be the same thing.

# 5. 5. What Should Be Logged?

Logs should be divided into well-defined categories.

## 5.1 5.1 Test Framework Logs

Generated by pytest or the test framework.

Examples:

TEST_START
TEST_STEP
TEST_PASS
TEST_FAIL
TEST_SKIP
TEST_ERROR
TEST_END

Example:

10:21:15.120 INFO  TEST_START
10:21:15.124 INFO  test_id=SOC_UART_001
10:21:15.125 INFO  step="Reset DUT"
10:21:17.342 INFO  step="Wait for boot"
10:21:18.002 INFO  step="Send UART command"
10:21:18.103 INFO  step="Validate response"
10:21:18.104 INFO  TEST_PASS

## 5.2 5.2 Test Action Logs

Every significant action performed by the automation framework should be logged.

Examples:

- Power ON

- Power OFF

- Reset

- Flash firmware

- Open UART

- Close UART

- Send command

- Read response

- Configure peripheral

- Read register

- Write register

- Start measurement

- Stop measurement

- Attach debugger

- Detach debugger

Example:

ACTION
execution_id=EXE-20260918-001
test_id=SOC_UART_001
action=UART_SEND
interface=UART0
data="AT+STATUS"

## 5.3 5.3 SoC Interface Logs

These capture communication with the DUT.

### 5.3.1 UART

Recommended fields:

timestamp
interface
direction
data
baudrate

Example:

12:31:02.120 UART0 TX  "status\r\n"
12:31:02.145 UART0 RX  "STATUS: READY\r\n"

### 5.3.2 SPI

Example:

SPI0
TX: 9F 00 00 00
RX: EF 40 18 00

### 5.3.3 I2C

Example:

I2C0
ADDR=0x50
WRITE=00 10
READ=AB CD

### 5.3.4 JTAG/SWD

Examples of events:

connect
reset
halt
resume
register read
memory read
memory write

## 5.4 5.4 Firmware Logs

Firmware-generated messages should be preserved separately from automation logs.

Example:

[BOOT] Starting system
[CLK] PLL configured
[MEM] DDR initialization OK
[UART] Driver initialized
[APP] Application started

The source of the message should be retained:

source = DUT
interface = UART0

Do not simply merge firmware output into the Python log and discard its origin.

## 5.5 5.5 Hardware / Board Logs

If the test infrastructure controls the physical board, capture:

- power supply state;

- voltage;

- current;

- temperature;

- reset state;

- relay state;

- board controller messages;

- power-cycle events;

- external instrumentation.

Example:

BOARD_POWER_ON
voltage=3.30V
current=142mA
timestamp=2026-09-18T12:31:00.123Z

For long-running tests, this information can be extremely valuable.

## 5.6 5.6 Environment Information

Every execution should capture the environment in which the test was executed.

Example:

execution:
  execution_id: EXE-20260918-001
  timestamp: 2026-09-18T12:30:00Z

test:
  suite: regression
  test_id: SOC_UART_001
  test_version: abc1234

dut:
  device: XYZ123
  silicon_revision: B1
  serial_number: DUT-00017
  board_revision: REV_C

firmware:
  version: 2.4.1
  git_commit: 8f23ab1
  build_id: BUILD-12345

environment:
  runner: Jenkins
  runner_id: runner-04
  python_version: 3.12
  pytest_version: 8.x
  os: Ubuntu 24.04

tools:
  debugger: J-Link
  debugger_version: x.x.x

This information is critical for failure reproduction.

A failure six months later is almost impossible to reproduce if the only information available is:

test_uart FAILED

# 6. 6. Execution ID

The system should introduce an Execution ID as the primary correlation mechanism.

Example:

EXE-20260918-153045-7F32

Everything generated during that execution should reference it.

Execution
   |
   +-- Test Case 001
   |    +-- Python log
   |    +-- UART log
   |    +-- JTAG log
   |    +-- screenshots
   |
   +-- Test Case 002
   |    +-- Python log
   |    +-- UART log
   |
   +-- Test Case 003
        +-- Python log
        +-- UART log
        +-- power measurements

The Execution ID should remain stable for the entire test run.

# 7. 7. Test ID

Each test should have a stable identifier.

Examples:

SOC-BOOT-001
SOC-UART-001
SOC-SPI-014
SOC-INT-023
SOC-PWR-007

Avoid using the Python function name as the only identifier.

For example:

def test_uart_command():
    ...

can be renamed.

Instead, the test should have a stable logical identifier:

@pytest.mark.test_id("SOC-UART-001")
def test_uart_command():
    ...

The stable Test ID can then be used for historical analysis.

# 8. 8. Step ID

Complex tests should also use explicit step identifiers.

Example:

TEST: SOC-UART-001

STEP-001  Power on DUT
STEP-002  Flash firmware
STEP-003  Reset DUT
STEP-004  Wait for boot
STEP-005  Send command
STEP-006  Validate response
STEP-007  Power off DUT

A failure can then be reported as:

test_id=SOC-UART-001
step_id=STEP-005
status=FAILED

instead of simply:

FAILED somewhere in test_uart.py

# 9. 9. Correlation ID

For complex operations, an additional correlation_id should be supported.

For example:

execution_id
test_id
step_id
correlation_id
timestamp

This allows several low-level events to be associated with one high-level operation.

Example:

CORR-000123
    |
    +-- Python: send command
    +-- UART: TX
    +-- DUT: command received
    +-- SPI: transaction
    +-- UART: RX
    +-- Python: response validated

# 10. 10. Log Entry Structure

The canonical machine-readable format should be JSON Lines (JSONL).

One event should correspond to one JSON object.

Example:

{
  "timestamp": "2026-09-18T12:31:02.120Z",
  "execution_id": "EXE-20260918-001",
  "test_id": "SOC-UART-001",
  "source": "test_framework",
  "level": "INFO",
  "event": "TEST_START",
  "message": "UART test started"
}

Another example:

{
  "timestamp": "2026-09-18T12:31:05.120Z",
  "execution_id": "EXE-20260918-001",
  "test_id": "SOC-UART-001",
  "source": "uart",
  "interface": "UART0",
  "direction": "TX",
  "event": "DATA",
  "data": "status\\r\\n"
}

And:

{
  "timestamp": "2026-09-18T12:31:05.145Z",
  "execution_id": "EXE-20260918-001",
  "test_id": "SOC-UART-001",
  "source": "uart",
  "interface": "UART0",
  "direction": "RX",
  "event": "DATA",
  "data": "STATUS: READY\\r\\n"
}

# 11. 11. Recommended Event Schema

A common event schema should be defined for all log sources.

Recommended mandatory fields:

| Field | Description |
| --- | --- |
| timestamp | Event timestamp in UTC |
| execution_id | Unique execution identifier |
| source | Component that generated the event |
| level | Severity |
| event | Event type |

Recommended contextual fields:

| Field | Description |
| --- | --- |
| test_id | Test case identifier |
| step_id | Test step identifier |
| correlation_id | Operation correlation identifier |
| interface | DUT interface |
| dut_id | DUT identifier |
| board_id | Board identifier |
| message | Human-readable description |
| data | Raw or structured event data |

Optional fields should be added based on the source.

# 12. 12. Log Levels

Log levels should be defined centrally.

| Level | Purpose |
| --- | --- |
| TRACE | Extremely detailed diagnostic information |
| DEBUG | Developer/debug information |
| INFO | Normal execution information |
| WARNING | Unexpected but non-fatal condition |
| ERROR | Test/framework error |
| CRITICAL | Infrastructure/system failure |

Example:

INFO     Flashing firmware
INFO     Firmware flash completed
DEBUG    Waiting for boot message
INFO     DUT boot completed
WARNING  Boot took longer than expected
ERROR    UART response timeout

# 13. 13. Raw vs Processed Logs

The logging architecture should follow this principle:

Never destroy the raw evidence.

Recommended flow:

+---------------+
                    | UART hardware |
                    +-------+-------+
                            |
                     raw byte stream
                            |
                 +----------v----------+
                 |   Raw Log Store    |
                 +----------+----------+
                            |
                          parser
                            |
                 +----------v----------+
                 | Structured Events  |
                 +----------+----------+
                            |
                     analysis engine
                            |
                 +----------v----------+
                 | Test Result/Report |
                 +---------------------+

If the parser has a bug, raw logs can be reprocessed.

If only parsed output is stored, the original evidence is lost.

# 14. 14. Storage Architecture

For the initial implementation, a filesystem-based artifact structure is sufficient.

A centralized logging platform such as Elasticsearch/Kibana should only be introduced if log volume and search requirements justify it.

Recommended architecture:

CI Runner
                       |
                       v
                Execution directory
                       |
       +---------------+----------------+
       |               |                |
       v               v                v
   raw_logs/       events/          artifacts/
       |               |                |
       v               v                v
   UART.log        events.jsonl      firmware.bin
   JTAG.log        results.json      dump.bin
   DUT.log                           trace.csv

All files should be associated with the same Execution ID.

# 15. 15. Recommended Execution Directory Structure

EXE-20260918-001/
|
+-- execution.json
+-- metadata.json
+-- result.json
|
+-- logs/
|   |
|   +-- framework/
|   |   +-- framework.log
|   |
|   +-- dut/
|   |   +-- uart0.log
|   |   +-- uart1.log
|   |
|   +-- debug/
|   |   +-- jtag.log
|   |
|   +-- board/
|       +-- board-controller.log
|
+-- events/
|   +-- events.jsonl
|
+-- measurements/
|   +-- power.csv
|   +-- temperature.csv
|   +-- performance.csv
|
+-- dumps/
|   +-- memory.bin
|   +-- registers.json
|
+-- reports/
|   +-- junit.xml
|   +-- report.html
|
+-- artifacts/
    +-- firmware.bin

This structure is easy to archive as a CI artifact.

# 16. 16. Long-Term Storage

The storage architecture should distinguish between large artifacts and searchable metadata.

Execution
                     |
          +----------+----------+
          |                     |
          v                     v
   Artifact Storage       Result Database
          |                     |
   Raw logs/files         Test results
   Dumps                   Metadata
   Measurements            Statistics

## 16.1 16.1 Artifact Storage

Stores large objects:

- raw UART logs;

- JTAG logs;

- firmware;

- memory dumps;

- traces;

- screenshots;

- power measurements.

## 16.2 16.2 Result Database

Stores searchable metadata:

execution_id
test_id
status
timestamp
DUT
board_revision
silicon_revision
firmware
git_commit
duration
failure_category
artifact_location

Large raw UART/JTAG logs should generally remain in artifact storage rather than being stored directly in a relational database.

# 17. 17. Database Model

A simple relational model can be used initially.

## 17.1 17.1 EXECUTION

EXECUTION
---------
execution_id PK
start_time
end_time
status
runner
branch
commit
firmware
dut_id
board_revision
silicon_revision

## 17.2 17.2 TEST_EXECUTION

TEST_EXECUTION
--------------
test_execution_id PK
execution_id FK
test_id
status
start_time
end_time
duration
failure_category
log_location

## 17.3 17.3 DUT

DUT
---
dut_id PK
device
serial_number
silicon_revision
board_revision

## 17.4 17.4 ARTIFACT

ARTIFACT
--------
artifact_id PK
execution_id FK
type
path
size
checksum

## 17.5 17.5 EVENT

An event table may be introduced later:

EVENT
-----
event_id
execution_id
test_execution_id
timestamp
source
event_type
severity
payload

However, every raw UART byte does not necessarily need to be stored as a database row.

# 18. 18. Failure Classification

Test results should contain more than a simple PASS/FAIL verdict.

Recommended failure categories:

TEST_FAILURE
ASSERTION_FAILURE
TIMEOUT
UART_FAILURE
JTAG_FAILURE
FLASH_FAILURE
BOOT_FAILURE
DUT_CRASH
DUT_HANG
POWER_FAILURE
COMMUNICATION_FAILURE
INFRASTRUCTURE_FAILURE
ENVIRONMENT_FAILURE
UNKNOWN

Example regression summary:

Regression summary

Total:                 2450
PASS:                  2301
FAIL:                    91
INFRA_FAILURE:           32
TIMEOUT:                 18
BOOT_FAILURE:            14
UART_FAILURE:             9
DUT_CRASH:                7
OTHER:                   11

This is significantly more useful than a simple pass/fail count.

# 19. 19. Failure Analysis Pipeline

The system should contain an automated analysis stage.

Raw Logs
   |
   v
Parser
   |
   v
Event Normalization
   |
   v
Failure Detection
   |
   +-- Timeout
   +-- Assertion
   +-- Boot failure
   +-- Crash
   +-- Communication
   +-- Infrastructure
          |
          v
    Failure Summary
          |
          v
      Test Report

Example failure report:

TEST FAILED

Test:
SOC-UART-014

Failure:
UART response timeout

Expected:
"READY"

Observed:
No response

DUT state:
Boot completed

Last DUT message:
"Initializing UART driver..."

Likely category:
DUT / UART initialization

Artifacts:
UART log
JTAG log
Firmware
Memory dump

# 20. 20. Correlation Between Logs

All log sources should be correlatable.

Example:

12:15:01.001  Python   Sending command
12:15:01.004  UART     TX: GET_STATUS
12:15:01.010  DUT      Processing command
12:15:01.011  SPI      Transaction started
12:15:01.020  SPI      Transaction completed
12:15:01.022  UART     RX: STATUS=OK
12:15:01.023  Python   Assertion passed

The events should be correlated through:

execution_id
test_id
step_id
correlation_id
timestamp

For high-resolution timing analysis, timestamps should use a consistent clock source where possible.

# 21. 21. Test Steps

Explicit test steps are recommended for all complex tests.

Example:

TEST: SOC-UART-001

STEP-001  Power on DUT
STEP-002  Flash firmware
STEP-003  Reset DUT
STEP-004  Wait for boot
STEP-005  Send command
STEP-006  Validate response
STEP-007  Power off DUT

A log entry can therefore look like:

{
  "timestamp": "2026-09-18T12:31:05.120Z",
  "execution_id": "EXE-20260918-001",
  "test_id": "SOC-UART-001",
  "step_id": "STEP-005",
  "source": "test_framework",
  "level": "INFO",
  "event": "STEP_START",
  "message": "Send UART command"
}

# 22. 22. CI Integration

The CI pipeline should follow this general flow:

Checkout
   |
   v
Build Firmware
   |
   v
Provision DUT
   |
   v
Execute Tests
   |
   v
Collect Logs
   |
   v
Parse Logs
   |
   v
Generate JUnit
   |
   v
Generate Test Report
   |
   v
Archive Artifacts

For example:

Jenkins
   |
   +-- test-results.xml
   +-- execution.json
   +-- logs/
   +-- measurements/
   +-- artifacts/

JUnit should be used as the standard CI test-result interface, while the richer logging format contains the detailed SoC information.

# 23. 23. Reporting

Execution logging and report generation should be separate concerns.

Test Execution
                       |
                       v
                 Logging System
                       |
          +------------+------------+
          |            |            |
          v            v            v
        JUnit        JSONL       Raw Logs
          |            |            |
          +------------+------------+
                       |
                       v
                   Analyzer
                       |
              +--------+--------+
              |                 |
              v                 v
         CI Dashboard       Quarto Report

Reports should support:

- overall execution summary;

- pass/fail/skip counts;

- failure categories;

- failed test details;

- test duration;

- DUT information;

- firmware version;

- build/commit information;

- links to raw logs;

- links to measurements;

- links to dumps and other artifacts.

# 24. 24. Retention Policy

Retention requirements should be explicitly defined.

Example:

| Artifact | Example Retention |
| --- | --- |
| Test result metadata | 1–2 years |
| Failed test logs | 1 year |
| Successful raw logs | 30–90 days |
| Firmware/build metadata | 1–2 years |
| Crash dumps | 1 year |
| Regression reports | 1–2 years |
| Debug traces | 30 days |

The exact values should be agreed with the project.

The retention policy should also define:

- maximum log size;

- compression;

- archive format;

- deletion policy;

- ownership;

- storage limits.

# 25. 25. Log Rotation

Long-running SoC tests can generate very large logs.

Log rotation should therefore be supported.

Example:

uart.log
uart.log.001
uart.log.002
uart.log.003
uart.log.current

Rotation can be triggered by:

- file size;

- elapsed time;

- test execution;

- DUT reboot.

Example policy:

Maximum file size: 100 MB
Compression: gzip

The actual limits should be configurable.

# 26. 26. Logging Performance

Logging itself must not significantly influence test behavior.

This is particularly important for:

- timing tests;

- interrupt tests;

- performance tests;

- high-speed communication;

- power-management tests.

Recommended requirements:

Logging shall be asynchronous where required.

Logging shall not block the test execution thread.

Logging overhead shall be measurable.

Raw high-frequency data shall support buffering/batching.

Logging configuration shall allow high-volume logging to be disabled.

For example:

LOG_LEVEL=ERROR

may be appropriate for a performance-sensitive test, while:

LOG_LEVEL=TRACE

may be used during debugging.

# 27. 27. Security and Sensitive Data

The logging system must define what information is permitted in logs.

Potentially sensitive information includes:

- credentials;

- authentication tokens;

- private keys;

- production secrets;

- customer information;

- proprietary information.

The logger should support masking.

Example:

username=test_user
password=********
token=********

Sensitive information should not be written to raw logs unless explicitly required and appropriately protected.

# 28. 28. Configuration

Logging should be configurable without modifying test implementation.

Example:

logging:
  level: INFO

  outputs:
    console: true
    file: true
    json: true

  interfaces:
    uart:
      enabled: true
      raw: true

    jtag:
      enabled: true

  rotation:
    max_size_mb: 100
    compression: true

CI can use:

INFO

while debugging can use:

DEBUG

or:

TRACE

# 29. 29. Example End-to-End Execution

A complete execution example should be included in the final implementation documentation.

Example metadata:

Execution ID:
EXE-20260918-001

DUT:
SOC-00017

Board:
REV-C

Silicon:
B1

Firmware:
2.4.1
commit 8f23ab1

Test:
SOC-UART-014

Execution:

15:02:01  Power ON
15:02:02  Reset DUT
15:02:04  Boot detected
15:02:05  UART initialized
15:02:06  Send command STATUS
15:02:06  RX STATUS=READY
15:02:06  Assertion PASS
15:02:07  Test PASS

Stored artifacts:

EXE-20260918-001/
|
+-- metadata.json
+-- events.jsonl
+-- logs/
|   +-- framework.log
|   +-- uart0.log
|   +-- jtag.log
|
+-- measurements/
|
+-- reports/
|   +-- junit.xml
|
+-- artifacts/

# 30. 30. Recommended Initial Architecture

For a Python/pytest-based SoC automation environment, the recommended initial architecture is:

pytest
                    |
                    v
             Logging Manager
                    |
        +-----------+------------+
        |           |            |
        v           v            v
     Framework     UART       JTAG/SWD
       events      driver       driver
        |           |            |
        +-----------+------------+
                    |
                    v
              Event Collector
                    |
          +---------+----------+
          |                    |
          v                    v
     Raw log files          JSONL
          |                    |
          +---------+----------+
                    |
                    v
              Test Analyzer
                    |
          +---------+----------+
          |         |          |
          v         v          v
       JUnit      JSON DB    Quarto
                    |
                    v
              Historical
               Analysis

# 31. 31. Recommended Technology Choices

| Component | Initial Choice |
| --- | --- |
| Test framework | pytest |
| Logging API | Python logging + custom structured logger |
| Canonical event format | JSONL |
| Raw interface logs | Plain text / binary where appropriate |
| Test result | JUnit XML |
| Metadata | JSON |
| Measurements | CSV/JSON |
| Local storage | Filesystem |
| CI storage | Jenkins/GitLab artifacts |
| Historical data | PostgreSQL/SQLite initially |
| Reporting | Quarto |
| Analysis | Python/pandas |
| Visualization | Quarto/Plotly |
| Compression | gzip/zstd |

The initial implementation should avoid unnecessary infrastructure complexity.

The event model, correlation model, metadata model, directory structure, and failure classification should be designed correctly first. A centralized logging platform can be introduced later without changing the fundamental test architecture.

# 32. 32. Key Design Principles

The following principles should be treated as architectural decisions.

## 32.1 32.1 Preserve Raw Evidence

Raw evidence shall never be destroyed as a result of log processing.

Raw logs must remain available even after parsing and analysis.

## 32.2 32.2 Every Event Must Be Traceable

Every event should be traceable to:

Execution
   |
   +-- DUT
   |
   +-- Test
        |
        +-- Step
             |
             +-- Event

## 32.3 32.3 Test Results and Logs Are Different Things

A test result answers:

Did the test pass or fail?

A log answers:

What happened during execution?

The two should be linked but not treated as the same data.

## 32.4 32.4 Logs Must Support Failure Reconstruction

The system should provide enough information to reconstruct a failure without requiring access to the original test runner session.

## 32.5 32.5 Logging Must Be Configurable

Different test types have different logging requirements.

The system should support at least:

ERROR
WARNING
INFO
DEBUG
TRACE

and source-specific configuration.

## 32.6 32.6 Logging Must Be Low Overhead

Logging must not materially alter the behavior being tested.

This is particularly important for timing- and performance-sensitive SoC validation.

# 33. 33. Final Architectural Principle

The most important principle for the overall design is:

A test execution log shall contain sufficient information to reconstruct and diagnose the execution without requiring access to the original test runner session.

This leads to the second key principle:

Raw evidence shall be preserved independently from processed and analyzed results.

Together, these principles should drive the decisions regarding event structure, correlation IDs, storage, retention, analysis, and reporting.

# 34. 34. Recommended Final Document Structure

The official engineering design document can be organized as follows:

1. Introduction
   1.1 Purpose
   1.2 Scope
   1.3 Definitions
   1.4 References

2. Requirements
   2.1 Functional Requirements
   2.2 Non-Functional Requirements
   2.3 Performance Requirements
   2.4 Retention Requirements

3. Current Test Environment
   3.1 Test Framework
   3.2 DUT
   3.3 Test Interfaces
   3.4 CI Infrastructure

4. Logging Architecture
   4.1 High-Level Architecture
   4.2 Logging Components
   4.3 Data Flow
   4.4 Execution Lifecycle

5. Logging Model
   5.1 Execution ID
   5.2 Test ID
   5.3 Step ID
   5.4 Correlation ID
   5.5 Log Levels
   5.6 Event Types

6. Log Sources
   6.1 Test Framework
   6.2 Automation Framework
   6.3 UART
   6.4 SPI/I2C
   6.5 JTAG/SWD
   6.6 Firmware
   6.7 Board Controller
   6.8 Measurements

7. Log Format
   7.1 Raw Logs
   7.2 Structured Logs
   7.3 Metadata
   7.4 Event Schema
   7.5 Examples

8. Log Collection
   8.1 Local Execution
   8.2 CI Execution
   8.3 Buffering
   8.4 Rotation
   8.5 Failure Handling

9. Storage
   9.1 Local Storage
   9.2 CI Artifacts
   9.3 Long-Term Storage
   9.4 Database
   9.5 Artifact Management

10. Log Processing
    10.1 Parsing
    10.2 Normalization
    10.3 Correlation
    10.4 Failure Detection
    10.5 Failure Classification

11. Analysis
    11.1 Test Result Analysis
    11.2 Failure Analysis
    11.3 Regression Analysis
    11.4 Trend Analysis
    11.5 Flaky Test Detection

12. Reporting
    12.1 CI Results
    12.2 JUnit
    12.3 Quarto
    12.4 Dashboards

13. Retention and Lifecycle
    13.1 Retention
    13.2 Archiving
    13.3 Compression
    13.4 Deletion

14. Security
    14.1 Access Control
    14.2 Sensitive Information
    14.3 Integrity

15. Performance
    15.1 Logging Overhead
    15.2 High-Frequency Logging
    15.3 Long-Running Tests

16. Error Handling

17. Configuration

18. Implementation Plan

19. Open Questions / Decisions

20. Appendices
    A. JSON Schema
    B. Example Execution
    C. Example Logs
    D. Database Schema